5kW 2000 rpm  PMSM Motor

| Basic Motor Parameters |  |  |  |  |
| --- | --- | --- | --- | --- |
| Rated Power | W | 5000 |  |  |
| Rated Speed | rpm | 2036 |  |  |
| Rated Torque | Nm | 24 |  |  |
| Rated Current | A | 115 |  |  |
| Rated Voltage | V | 48 |  |  |
| No-Load Speed | rpm | 2100 |  |  |
| Max. Efficiency | % | 92 |  |  |
| Electrical Parameters |  |  |  |  |
| Number of Phases |  | 3 |  |  |
| Number of Poles |  | 6 |  |  |
| Torque Constant (KT) | Nm/A | 0.20 |  |  |
| Back EMF Constant (KE) | Vs/rad | 0.19 |  |  |
| Terminal Resistance Phase to Phase (±%10) | mΩ | 11.92 |  |  |
| Terminal Inductance Phase to Phase (±%30) | mH | 0.103 |  |  |
| Thermal Data |  |  |  |  |
| Ambient Temperature | °C | -40...+85 |  |  |
| Max. Winding Temperature | °C | +155 |  |  |
| Physical Parameters |  |  |  |  |
| Outer Diameter | mm | 214 |  |  |
| Shaft Diameter | mm | 25 |  |  |
| Max. Lenght | mm | 285 |  |  |
| Weight | gr | ~20000 |  |  |
| Motor Connection |  |  |  |  |
| U | SIHO-S55 CABLE |  |  |  |
| V | SIHO-S55 CABLE |  |  |  |
| W | SIHO-S55 CABLE |  |  |  |
| Sensor Connection |  |  |  |  |
| N/A | N/A |  |  |  |
| Connector Articles |  |  |  |  |
| Power | N/A |  |  |  |
| Signal | N/A |  |  |  |